Lanif98 1.0.0.x -Readme

Warning

Lanif98 may only be used by authorised subscribers.

Installation

Lanif98 is supplied in a self-extracting zip executable – Lanif98se.exe. Having downloaded Lanif98se.exe, locate the program in your download directory and double click on it to start the self extraction process.

Unless you are an advanced PC user, accept the default "Unzip To" folder of C:\Program Files\Dicam\Lanif. Do not run Winzip, even if it is installed on your computer.

When extraction is completed, Wordpad should open this file, Readme.doc.

Running Lanif98

Using Windows Explorer, locate the Lanif98.exe program in C:\Program Files\Dicam\Lanif and double click on it

or

In the Start menu choose Run..    C:\Program Files\Dicam\Lanif\Lanif98.exe

For easier access to the program –

Using Windows Explorer, locate the Lanif98.exe program in C:\Program Files\Dicam\Lanif and drag the program onto your desktop or onto your Start button. (This doesn't move the program, it just creates a shortcut.)

The first time you run Lanif98, you must configure you modem and file paths -

Configuring Lanif98

Select Site | Properties to configure modem and paths.

Modem configuration

Modem: Select the modem you will use for connection to Dicam control networks.

Configure... Configures the modem. We suggest the following modem settings for optimal connection throughput:

General | Maximum speed: 4800

Connection |

Data bits: 8

Parity: None

Stop bits: 1

Connection | Port settings... | Use FIFO buffers: off (unchecked)

Connection | Advanced... |

Use error control: off

Use flow control: on, Hardware

Modulation type: Standard

Dial prefix: make an entry here if you need a dial prefix such as 9 to get an outside line.

Timeout: Specifies dialling and connection timeout in seconds. A setting here means that Lanif98 will automatically drop the call if the connection fails. The default setting of 0 seconds means that no timeout is applied – it will stay on line until you cancel the call.

Local port: Specifies a serial port for direct connection to a Dicam interface unit through a "null-modem" cable.

Paths configuration

Site(s) directory: Specifies the root folder for .STE and supplementary files. For example C:\DicamData. If you are connecting to several sites, create a separate folder for each site e.g. C:\DicamData\Site1 and so on.

Dialling sites

Site Access

Lanif98 permits access to multiple sites, subject to access restrictions. For security, each individual site has a Site Identifier - a number like 12345, which usually corresponds to the serial number of the Netmon or Logger unit. To connect to a site you need a site configuration file (.STE – such as 12345.STE) and a corresponding site access key. (See Further Information below.) The site configuration file (.STE) contains information about the site such as the telephone number and details of the network units. The site access key is a security measure to regulate authorised connection to the site.

To enter your site access key, use Help | Site Access

Opening a site

Use the Open Folder icon or select Site | Open...

Browse directories as required and select a site configuration (.STE) file.

Modem connection

To dial a site, use the "Lift Receiver" icon or select Site | Connect Remote. This opens the automatic dial feature. The program automatically dials the site and attempts to make a software connection to the Dicam network, and meanwhile the box shows state of progress. Once it has made a connection, the box shows "Connected" and the bottom bar show "Online". If the attempt is unsuccessful, or if you want to abort the dialling attempt, select Cancel.

Note : check the telephone number shown in the "Telephone" box. This shows the number it is going to dial. (Extra characters may be dialled first if you have set up a dial prefix – see above).

The number may include long distance dialling codes before the local number which you may want to edit out first. It may also include commas and extra numbers – these are for access through a line switcher. Don't alter these.)

For more information on features available during a connection, see additional documentation.

To hang up the call, press the "Receiver Down" icon.

To Close the Program

Press the "Exit" icon in the top right of the screen.

Further information

Sites can only be accessed by authorised users using encoded site access keys. For site access keys and other information regarding site configuration, make a note of the site identifier(s) and email this with your request to: nick@farmex.com

For further documentation and program updates, please check the Dicam website: 	http://www.dicam.co.uk

and/or email: support@dicam.co.uk

Before checking for updates or contacting support, make a note of the version number of your existing copy of Lanif98. Find this from the Help | About dialogue or by right-clicking on Lanif98.exe, select "Properties" and then "Version".